SILO CONTROLLER

Patrick Blackett — Silo 2 Controller

Technical Reference & User Manual

April 2026

# Introduction

The Silo Controller is a Raspberry Pi 5 application designed to control a linear actuator that opens and closes a silo lid fitted to a vessel. It simultaneously records navigation and attitude telemetry from an ArduPilot autopilot via MAVLink, records video from a Pi Camera with a live telemetry overlay, and provides control via a web browser, MQTT broker, and HTTP REST API.

The system is designed to run headlessly under systemd with all operator interaction through a browser at port 5000. An optional Tk desktop GUI is available when a display is connected.

## Scope

Control the silo lid (open/close) via relay-driven linear actuator

Record MAVLink telemetry to CSV files

Stream and record Pi Camera video with telemetry overlay burned in

Accept commands and publish state via MQTT

Provide a full REST HTTP API

Configurable travel time, persistent relay state, and stress-test facility

# Hardware

## Bill of Materials

| Item | Part | Notes |
| --- | --- | --- |
| Compute | Raspberry Pi 5 | Main controller |
| Relay | Eletechsup DR25E01 | 3 A, 1-ch, bistable DPDT, 5 V logic |
| Actuator | Linak LA 121P00-1101220 | 12 V DC, internal end-stop switches |
| Autopilot/FC | Matek H743 | ArduPilot, USB serial to Pi |
| GPS | u-blox M10Q | Via ArduPilot (GPS1_TYPE=9). Supercapacitor backed. |
| RF Link | mLRS | MAVLink over radio |
| Camera | Pi Camera Module | Optional — auto-detected |
| Power (motor) | 12 V PSU | Separate from Pi logic supply |

## GPIO Wiring (BCM numbering)

The DR25E01 bistable relay uses a single trigger input (T). It toggles to the opposite position on each LOW pulse and latches without power.

| Pi Pin | DR25E01 Pin | Function |
| --- | --- | --- |
| GPIO 17 | T (Trigger) | 100 ms LOW pulse toggles relay |
| 5 V pin | V (Coil power) | Held HIGH at idle |
| GND | G (Ground) |  |

Relay OUT+/OUT− connect to the actuator motor terminals. If open/close direction is reversed, swap OUT+/OUT−. A separate 12 V supply powers the actuator through the relay.

## How the Bistable Relay Works

The relay has no position feedback output. The software tracks which position the relay is in (relay_open bool) and only pulses pin T when a direction change is required. This state is persisted to silo_state.json and survives reboots. If the physical relay ever gets out of sync with the software state (e.g. after a power cut mid-travel), use the Configuration tab in the web UI to declare the actual position.

Idle:    GPIO 17 HIGH  — relay holds its last position (no power needed)
Toggle:  GPIO 17 LOW for 100 ms  — relay flips, then GPIO 17 returns HIGH

# Software Architecture

The application is written in Python 3 and structured as a set of independent modules, each running on daemon threads. main.py is the single entry point.

## Module Overview

| Module | File | Role |
| --- | --- | --- |
| SiloController | core/silo.py | State machine + GPIO driver. Thread-safe open/close/declare. |
| TelemetryReader | telemetry/reader.py | MAVLink reader; auto-detects serial port; provides live frame + diagnostics. |
| Recorder | telemetry/recorder.py | CSV telemetry logger. One file per session. PB*.csv naming. |
| StatsTracker | telemetry/stats.py | Rolling 1/5/10/30 min min/avg/max statistics. |
| CameraRecorder | camera/recorder.py | Pi camera auto-detect, H264 MP4 recording, MJPEG stream, telem overlay. |
| Flask app | web/app.py | REST API + SSE at port 5000. |
| MqttHandler | mqtt/handler.py | Paho MQTT client. Commands in, state out, events out. |
| SiloGUI | gui/app.py | Optional Tk desktop GUI (skipped if no display). |

## Thread Model

main thread — Tk event loop (or signal.pause() headless)

mavlink-reader — serial MAVLink reader loop

web-server — Flask threaded server

sse-telem — pushes telemetry to SSE clients at 5 Hz

sse-hb — heartbeat every 5 s

actuator-open / actuator-close — short-lived per move

stress-test — long-lived during stress test only

cam-capture — continuous MJPEG frame capture (if camera present)

# Web Interface

Navigate to http://<Pi-IP>:5000 in any browser on the same network.

## Tabs

### Live

Shows the current silo state (open/closed/opening/closing with animated dot), recording state, GPS diagnostics (fix type, satellites, HDOP, VDOP, H/V accuracy), mLRS link quality, navigation data (lat, lon, alt, speed, COG, pitch, roll), and a motion analysis table showing rolling statistics.

### Video

The second tab. Shows the live MJPEG camera feed with telemetry overlay burned into the image (time, speed, pitch, roll, COG, GPS fix). Telemetry data is also displayed in panels to the right of the feed. Features:

Military test-range aesthetic — dark background, gold borders, scanlines

● RECORDING / ○ LIVE indicator

Snapshot button — downloads JPEG immediately

Manual record start/stop

Custom status message input — text is burned into video overlay and logged

Saved video list with MP4 download

Telem panel: GPS fix, lat/lon/alt, satellites, speed, COG, pitch, roll, silo state

### History

Interactive track map (English Channel coastline), and scrollable/zoomable time-series charts for speed, heading, pitch, roll, pitch/roll/yaw rates, and altitude.

### Diagnostics

Full autopilot diagnostics including:

Autopilot firmware version, board UID, capabilities

System status, EKF health flags and variances

GPS signal detail (fix type, satellites, HDOP, VDOP, accuracy)

GPS module hardware (type, HW/SW version if captured at boot)

Satellite sky plot and per-satellite SNR table

mLRS radio link quality, RSSI, buffer, errors

Battery voltage, current, remaining %

FC status log (STATUSTEXT messages from ArduPilot)

### Log Files

Lists all CSV recording files with size, row count, and date. Clicking a file shows a full summary: start/end time, duration, motion statistics (min/avg/max for speed, pitch, roll, rates, altitude), GPS bounds, silo events, GPS fix quality histogram, and a mini track map. Direct CSV download.

### Configuration

Declare Silo Position — manually declare open or closed without moving the actuator

Actuator Travel Time — set the software timer (0.5–120 s) that matches actual stroke time

Stress Test — run N open/close cycles with configurable pause, live progress bar, stop button

Relay Wiring reference — GPIO 17, pulse width, state file location

### API Docs

Shows exact curl and mosquitto commands with the Pi's real IP address pre-filled. Every code block has a one-click Copy button. Covers silo control, recording, telemetry queries, SSE, MQTT topics, camera message API, and GPIO wiring.

# REST API

Base URL: http://<Pi-IP>:5000   No authentication required.

## Silo Control

| Method | Path | Description | Response |
| --- | --- | --- | --- |
| POST | /api/open | Open silo lid | {"ok": true} |
| POST | /api/close | Close silo lid | {"ok": true} |
| GET | /api/status | State + recording + uptime | {"state":"open","recording":false,"uptime_s":3612} |
| POST | /api/config/declare | Declare position {"open":true} | {"ok":true,"state":"open"} |
| GET | /api/config/relay_state | Relay open bool + state + travel_time | JSON |
| GET | /api/config/travel_time | Current travel time (s) | {"travel_time":10.0} |
| POST | /api/config/travel_time | {"travel_time":12.5} | {"ok":true,"travel_time":12.5} |

## Telemetry

| Method | Path | Description |
| --- | --- | --- |
| GET | /api/telemetry | Live telemetry frame (lat, lon, alt, speed, pitch, roll, GPS fix, etc.) |
| GET | /api/diagnostics | Full diagnostics dict including GPS frame fields, EKF, radio, battery |
| GET | /api/events | SSE stream: state, telemetry, recording, cam_recording, heartbeat |
| GET | /api/history | Historical telemetry rows for charts |
| GET | /api/track | GPS track points for the map |

## Camera

| Method | Path | Description |
| --- | --- | --- |
| GET | /api/camera/status | Camera detected + recording state |
| GET | /api/camera/stream | MJPEG live stream (open in browser or VLC) |
| GET | /api/camera/snapshot | JPEG download of current frame |
| POST | /api/camera/message | {"message":"text"} — add overlay annotation |
| GET | /api/camera/messages | All messages log |
| GET | /api/camera/videos | List saved MP4 files |
| GET | /api/camera/videos/<name>/download | Download a specific MP4 |
| POST | /api/camera/record/start | Manual recording start |
| POST | /api/camera/record/stop | Manual recording stop |

## Stress Test

| Method | Path | Description |
| --- | --- | --- |
| POST | /api/stress/start | {"cycles":10,"pause_s":3} — start automated cycle test |
| POST | /api/stress/stop | Abort stress test |
| GET | /api/stress/status | running, done, total, step |

## Example curl Commands

# Open the silo
curl -X POST http://192.168.1.100:5000/api/open

# Check status
curl http://192.168.1.100:5000/api/status

# Post a camera overlay message
curl -X POST http://192.168.1.100:5000/api/camera/message \
  -H 'Content-Type: application/json' \
  -d '{"message":"LAUNCH SEQUENCE ARMED"}'

# Download latest video
curl -O http://192.168.1.100:5000/api/camera/videos/PBvideo_20260412_031100.mp4/download

# Run 5-cycle stress test with 5 s pause
curl -X POST http://192.168.1.100:5000/api/stress/start \
  -H 'Content-Type: application/json' \
  -d '{"cycles":5,"pause_s":5}'

# MQTT Interface

A Mosquitto broker runs on the Pi at localhost:1883 with no authentication. The broker is accessible from any device on the same network.

## Installing the Broker

sudo apt install mosquitto mosquitto-clients
sudo systemctl enable --now mosquitto

## Topics

| Direction | Topic | Payload | Notes |
| --- | --- | --- | --- |
| Subscribe | silo/command | OPEN / CLOSE / STATUS | Any case accepted |
| Subscribe | silo/record | START / STOP / STATUS | CSV telemetry recording |
| Subscribe | silo/message | Any text | Posted as camera overlay message |
| Publish (retained) | silo/status | open / closed / opening / closing | Immediate on subscribe |
| Publish (retained) | silo/recording | true / false | Immediate on subscribe |
| Publish | silo/event | JSON {"event":…,"ts":…,"source":…} | Every state transition |

## Example mosquitto Commands

# Open silo
mosquitto_pub -h 192.168.1.100 -t silo/command -m OPEN

# Close silo
mosquitto_pub -h 192.168.1.100 -t silo/command -m CLOSE

# Post a status message (overlay on video)
mosquitto_pub -h 192.168.1.100 -t silo/message -m "TORPEDO DOOR OPEN"

# Monitor silo state changes
mosquitto_sub -h 192.168.1.100 -t silo/status

# Subscribe to all silo events with timestamps
mosquitto_sub -h 192.168.1.100 -t silo/event

# Example event payload:
{"event": "opening", "ts": "2026-04-12T03:11:00Z", "source": "web"}

## Silo Event Source Values

| Source | Triggered by |
| --- | --- |
| web | Browser UI button |
| mqtt | MQTT silo/command |
| api | HTTP REST call |
| config-ui | Configuration tab declare button |
| stress-test | Automated stress test |
| actuator | Internal — final open/closed state after travel |
| manual | Configuration tab declare |

# Camera and Video

## Auto-Detection

On startup the application calls Picamera2.global_camera_info(). If one or more cameras are detected the CameraRecorder initialises immediately. If no camera is found, all camera API endpoints return gracefully (204 No Content for stream/snapshot, JSON with camera_detected:false for status) and all other application functions continue normally.

## Automatic Recording

Recording starts automatically when the silo begins opening (state = opening) and stops 2 seconds after the silo reaches the closed state. This ensures the full open-to-close sequence is captured including the final moments of closing.

Recording can also be started and stopped manually via the Video tab, the REST API, or MQTT.

## Video Overlay

A telemetry overlay is burned into both the recorded MP4 files and the live MJPEG stream. The overlay always shows:

● REC (recording) or ○ LIVE (stream only) indicator

Current UTC date and time

Ground speed (knots)

Altitude (metres)

Pitch angle (degrees, +nose-up)

Roll angle (degrees, +starboard)

Course over ground (degrees)

GPS fix type and satellite count

Last 3 custom status messages with timestamps

## Status Messages

Custom text messages can be sent via HTTP POST, MQTT publish, or the Video tab input field. Messages are timestamped (UTC), stored in a rolling log (last 20 kept), and the most recent 3 are shown on the video overlay. They persist in memory until the service restarts.

System events are automatically posted as messages: when the silo begins opening, reaches open, begins closing, or reaches closed, a timestamped entry is added — e.g. 'SILO OPENING  [03:11:00 UTC]  via web'.

## Disk Management

Before starting each new recording, the application checks available disk space. If free space is below 15% of the total, the oldest MP4 file in recordings/video/ is deleted. This ensures the system never fills the SD card.

## Video File Format

| Parameter | Value |
| --- | --- |
| Container | MP4 |
| Video codec | H.264 |
| Resolution | 1280 × 720 |
| Frame rate | 30 fps |
| Bitrate | 4 Mbps |
| Naming | PBvideo_YYYYMMDD_HHMMSS.mp4 |
| Location | recordings/video/ |

# Silo State Machine

## States

| State | Meaning | Relay | Actuator |
| --- | --- | --- | --- |
| closed | Lid fully closed (default) | Retracted position | Stationary |
| opening | Lid moving open | Extended position | Extending |
| open | Lid fully open | Extended position | Stationary (limit switch) |
| closing | Lid moving closed | Retracted position | Retracting |

## Transitions

closed  →  opening  →  open
open    →  closing  →  closed

An open command when already open (or opening) is rejected with {"ok": false, "reason": "Already open"}. Similarly for close.

## Travel Time

The travel_time setting (default 2 s, configurable 0.5–120 s) controls how long the software waits in the opening/closing state before declaring the move complete. The Linak actuator's internal limit switches stop the motor at both ends automatically — travel_time is a software display timer, not a hard motor cutoff. Set it slightly longer than the actual full-stroke time.

## Stress Test

The stress test runs N open/close cycles with a configurable pause between each move. It waits for actual state transitions (polling silo.state) rather than sleeping — so it correctly handles any travel time setting. The test can be stopped at any point; the silo will complete its current move before stopping.

# Telemetry Recording

CSV files are written to recordings/ with the naming convention PB_YYYYMMDD_HHMMSS.csv. Each file covers one recording session (start/stop via UI, MQTT, or API).

## CSV Columns

Key fields written per row:

timestamp_unix — Unix timestamp (float)

timestamp_iso — ISO 8601 UTC string

lat, lon — decimal degrees

alt_m — altitude metres MSL

heading_deg — course over ground degrees

groundspeed_ms — speed m/s

pitch_deg, roll_deg, yaw_deg — attitude degrees

pitch_rate_deg_s, roll_rate_deg_s, yaw_rate_deg_s — angular rates

gps_fix — 0=none, 2=2D, 3=3D, 4=DGPS, 5=RTK float, 6=RTK fixed

satellites — satellite count

hdop, vdop — dilution of precision

silo_state, silo_event, silo_source — silo transitions when they occur

## GPS Assistance (Cold Start)

The script tools/gps_assist.py injects cold-start assistance data directly to the u-blox GPS via MAVLink GPS_INJECT_DATA messages, without any internet registration. This can reduce GPS acquisition time from ~15 minutes to 30–90 seconds outdoors.

Three types of data are injected:

Current UTC time (from Pi NTP-synced clock)

Approximate position (default: PO18 9AB, Bosham — configurable)

Current GPS almanac (YUMA format from NAVCEN USCG — free, no login)

python tools/gps_assist.py
python tools/gps_assist.py --lat 50.83 --lon -0.87 --port /dev/ttyACM1

# Installation

## Dependencies

# Python packages
pip3 install flask paho-mqtt gpiozero pymavlink
pip3 install picamera2 opencv-python  # optional — for camera

# System packages
sudo apt install mosquitto mosquitto-clients ffmpeg

## Running Manually

cd /home/pi/SiloController
python3 main.py

## Running as a systemd Service

[Unit]
Description=Silo Controller
After=network.target

[Service]
ExecStart=/usr/bin/python3 /home/pi/SiloController/main.py
WorkingDirectory=/home/pi/SiloController
Restart=always
RestartSec=3
User=pi

[Install]
WantedBy=multi-user.target

sudo systemctl enable silo
sudo systemctl start silo
sudo journalctl -u silo -f

## Checking Status

sudo systemctl status silo        # service health
sudo journalctl -u silo -n 50     # last 50 log lines
curl http://localhost:5000/api/status  # API health check

# Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Silo opens when it should close (or vice versa) | Relay OUT+/OUT− wired backwards | Swap OUT+ and OUT− on the actuator motor terminals |
| Software state out of sync with physical silo | Power cut during travel, or manual override | Use Configuration tab → Declare Silo Position |
| No GPS fix | Cold start, blocked antenna, or supercapacitor depleted | Run tools/gps_assist.py to inject almanac and time assistance |
| GPS fix/satellite count not shown in Diagnostics | Known: GPS frame data not in diagnostics dict before this fix | Ensure web/app.py merges frame data into /api/diagnostics response |
| MAVLink device not found | USB serial not detected, or wrong port | Check /dev/ttyACM* exists; plug in Matek via USB; check baud rate (57600) |
| Camera not detected | picamera2 not installed, or camera cable not seated | pip3 install picamera2; check ribbon cable; run Picamera2.global_camera_info() |
| MQTT commands not working | Mosquitto broker not running | sudo systemctl start mosquitto |
| Video overlay not visible | opencv-python not installed | pip3 install opencv-python |
| Disk full — video not recording | SD card full (< 15% free check triggered delete) | Delete old recordings manually, or increase SD card size |

# File Structure

SiloController/
├── main.py                    Entry point
├── CLAUDE.md                  AI context file for Claude Code
├── requirements.txt           Python dependencies
├── silo_state.json            Persistent relay state (auto-generated)
│
├── core/
│   └── silo.py                SiloController — state machine + GPIO
│
├── telemetry/
│   ├── reader.py              MAVLink reader + TelemetryFrame
│   ├── recorder.py            CSV recording
│   └── stats.py               Rolling statistics
│
├── camera/
│   └── recorder.py            Pi camera + MJPEG + MP4 + overlay
│
├── web/
│   ├── app.py                 Flask REST API + SSE
│   └── templates/
│       └── index.html         Single-page browser UI
│
├── mqtt/
│   └── handler.py             Paho MQTT client
│
├── gui/
│   └── app.py                 Optional Tk desktop GUI
│
├── tools/
│   └── gps_assist.py          GPS cold-start assistance injector
│
└── recordings/
    ├── PB*.csv                CSV telemetry files
    └── video/
        └── PBvideo_*.mp4      H264 video recordings